DATA DICTIONARY

Lysosome Analysis Script
Datasets & Column Guide

This document describes the CSV/tabular datasets generated by the script, what each dataset represents, and the meaning of every fixed column. It also explains the dynamically generated distance-to-puncta columns and the Napari-edited output. Definitions below follow the script comments and calculations; where a field is an internal identifier rather than a biological measurement, that distinction is stated explicitly.

# 1. Dataset overview

| Dataset | Role |
| --- | --- |
| lysosome_blobs_regions.csv | Initial lysosome/blob measurements before cell-versus-outside classification. |
| lysosome_blobs_regions_unique_radius.csv | Same lysosome measurements after duplicate radii are made slightly unique; diameter and volume are recalculated. |
| lysosome_counts_cell_vs_outside.csv | Two-category count summary: lysosomes classified as cell or outside. |
| lysosome_counts_by_cell.csv | Count of intracellular lysosomes for each original watershed cell ID. |
| lysosomes_with_cell_vs_outside.csv | Complete lysosome-level table after spatial cell assignment and per-cell lysosome serial numbering. |
| lysosome_counts_by_cell_serial.csv | Per-cell lysosome counts plus serial visualization cell ID and physical cell volume. |
| cell_signal_vs_lysosome_signal_extended.csv | Cell-level CH1 signal/volume partitioning among lysosome core, core+associated region, residual membrane, and distance-to-puncta bins. |
| lysosomes_with_cell_vs_outside_diameter_interval.csv | Diameter-filtered lysosome subset used for visualization and Napari editing. |
| cell_CH1_signal_percent_inside_mask.csv | Per-cell CH1 signal total, CH1 signal inside the original neuron mask, and percentage inside mask. |
| lysosomes_with_cell_vs_outside_EDITED.csv | Napari-edited lysosome table, auto-saved after interactive changes to classification/serial IDs. |

# 2. Lysosome-level datasets

## 2.1 Initial lysosome measurements

lysosome_blobs_regions.csv

One row per detected lysosome/blob. This is the first exported measurement table.

| Column | Type / unit | Meaning |
| --- | --- | --- |
| id | integer | Sequential lysosome/blob identifier created from the detected blobs. |
| z_um | µm | Physical Z coordinate of the detected lysosome center. |
| y_um | µm | Physical Y coordinate of the detected lysosome center. |
| x_um | µm | Physical X coordinate of the detected lysosome center. |
| radius_um | µm | Estimated lysosome radius in physical units. |
| diameter_um | µm | Estimated lysosome diameter; calculated as 2 × radius_um. |
| volume_um3 | µm³ | Estimated spherical lysosome volume, calculated as (4/3)πr³. |
| peak_gray | CH1 intensity | Local maximum intensity measured from raw channel 1. |
| slice | Z-slice index | Selected Z slice associated with the lysosome peak/localization. |

## 2.2 Unique-radius lysosome measurements

lysosome_blobs_regions_unique_radius.csv

Created when duplicate radii are present. The helper function shifts later duplicate radius values slightly while attempting to stay within ±5% and within the configured radius limits. Diameter and spherical volume are then recalculated.

| Column | Type / unit | Meaning |
| --- | --- | --- |
| id | integer | Sequential lysosome/blob identifier created from the detected blobs. |
| z_um | µm | Physical Z coordinate of the detected lysosome center. |
| y_um | µm | Physical Y coordinate of the detected lysosome center. |
| x_um | µm | Physical X coordinate of the detected lysosome center. |
| radius_um | µm | Estimated lysosome radius in physical units. |
| diameter_um | µm | Estimated lysosome diameter; calculated as 2 × radius_um. |
| volume_um3 | µm³ | Estimated spherical lysosome volume, calculated as (4/3)πr³. |
| peak_gray | CH1 intensity | Local maximum intensity measured from raw channel 1. |
| slice | Z-slice index | Selected Z slice associated with the lysosome peak/localization. |

Important: The biological detection is not repeated here; this table is a numerical adjustment of duplicate radius values for uniqueness.

## 2.3 Complete lysosome-to-cell assignment

lysosomes_with_cell_vs_outside.csv

The main classified lysosome table. It extends the measurement table with cell/outside classification and cell/lysosome identifiers.

| Column | Type / unit | Meaning |
| --- | --- | --- |
| id | integer | Sequential lysosome/blob identifier created from the detected blobs. |
| z_um | µm | Physical Z coordinate of the detected lysosome center. |
| y_um | µm | Physical Y coordinate of the detected lysosome center. |
| x_um | µm | Physical X coordinate of the detected lysosome center. |
| radius_um | µm | Estimated lysosome radius in physical units. |
| diameter_um | µm | Estimated lysosome diameter; calculated as 2 × radius_um. |
| volume_um3 | µm³ | Estimated spherical lysosome volume, calculated as (4/3)πr³. |
| peak_gray | CH1 intensity | Local maximum intensity measured from raw channel 1. |
| slice | Z-slice index | Selected Z slice associated with the lysosome peak/localization. |
| location_ch2 | text: cell/outside | Spatial classification based on channel-2/cell segmentation: “cell” for intracellular lysosomes and “outside” otherwise. |
| cell_id_ch2 | integer | Original watershed cell label assigned to the lysosome. Outside/unassigned lysosomes receive 0. |
| cell_id_ch2_viz | integer | Serial visualization cell ID obtained by mapping the original watershed cell ID to the visualization label set. Removed/nonvisualized cells receive 0. |
| lys_id_in_cell | integer | Serial lysosome number within its assigned cell. Intracellular lysosomes are sorted by cell_id_ch2, z_um, y_um, x_um and numbered from 1. Outside/unassigned lysosomes receive 0. |

## 2.4 Diameter-interval lysosome subset

lysosomes_with_cell_vs_outside_diameter_interval.csv

Copy of the classified lysosome table restricted to the optional minimum/maximum diameter selected by the user. The script recomputes interval-specific lys_id_in_cell values for the remaining intracellular lysosomes.

| Column | Type / unit | Meaning |
| --- | --- | --- |
| id | integer | Sequential lysosome/blob identifier created from the detected blobs. |
| z_um | µm | Physical Z coordinate of the detected lysosome center. |
| y_um | µm | Physical Y coordinate of the detected lysosome center. |
| x_um | µm | Physical X coordinate of the detected lysosome center. |
| radius_um | µm | Estimated lysosome radius in physical units. |
| diameter_um | µm | Estimated lysosome diameter; calculated as 2 × radius_um. |
| volume_um3 | µm³ | Estimated spherical lysosome volume, calculated as (4/3)πr³. |
| peak_gray | CH1 intensity | Local maximum intensity measured from raw channel 1. |
| slice | Z-slice index | Selected Z slice associated with the lysosome peak/localization. |
| location_ch2 | text: cell/outside | Spatial classification based on channel-2/cell segmentation: “cell” for intracellular lysosomes and “outside” otherwise. |
| cell_id_ch2 | integer | Original watershed cell label assigned to the lysosome. Outside/unassigned lysosomes receive 0. |
| cell_id_ch2_viz | integer | Serial visualization cell ID obtained by mapping the original watershed cell ID to the visualization label set. Removed/nonvisualized cells receive 0. |
| lys_id_in_cell | integer | Serial lysosome number within its assigned cell. Intracellular lysosomes are sorted by cell_id_ch2, z_um, y_um, x_um and numbered from 1. Outside/unassigned lysosomes receive 0. |

Important: If no diameter minimum/maximum is selected, this subset can contain the full classified table. This is the subset used for visualization and Napari editing.

## 2.5 Napari-edited lysosome table

lysosomes_with_cell_vs_outside_EDITED.csv

Auto-saved after the Napari editing session. It starts from the editable diameter-filtered table and writes back location_ch2, cell_id_serial, and lys_id_serial from the Napari point properties.

| Column | Type / unit | Meaning |
| --- | --- | --- |
| id | integer | Sequential lysosome/blob identifier created from the detected blobs. |
| z_um | µm | Physical Z coordinate of the detected lysosome center. |
| y_um | µm | Physical Y coordinate of the detected lysosome center. |
| x_um | µm | Physical X coordinate of the detected lysosome center. |
| radius_um | µm | Estimated lysosome radius in physical units. |
| diameter_um | µm | Estimated lysosome diameter; calculated as 2 × radius_um. |
| volume_um3 | µm³ | Estimated spherical lysosome volume, calculated as (4/3)πr³. |
| peak_gray | CH1 intensity | Local maximum intensity measured from raw channel 1. |
| slice | Z-slice index | Selected Z slice associated with the lysosome peak/localization. |
| location_ch2 | text: cell/outside | Spatial classification based on channel-2/cell segmentation: “cell” for intracellular lysosomes and “outside” otherwise. |
| cell_id_ch2 | integer | Original watershed cell label assigned to the lysosome. Outside/unassigned lysosomes receive 0. |
| cell_id_ch2_viz | integer | Serial visualization cell ID obtained by mapping the original watershed cell ID to the visualization label set. Removed/nonvisualized cells receive 0. |
| lys_id_in_cell | integer | Serial lysosome number within its assigned cell. Intracellular lysosomes are sorted by cell_id_ch2, z_um, y_um, x_um and numbered from 1. Outside/unassigned lysosomes receive 0. |
| cell_id_serial | integer | Serial cell identifier stored/edited in the Napari point properties. Invalid/missing values are normalized to 0 by attach_all_blob_fields(). |
| lys_id_serial | integer | Serial lysosome identifier stored/edited in the Napari point properties. Invalid/missing values are normalized to 0. |

Important: The exact final column set follows the editable DataFrame. attach_all_blob_fields() guarantees z_um, y_um, x_um, radius/diameter relationship, location_ch2, cell_id_ch2, and peak_gray when needed.

# 3. Lysosome count summary datasets

## 3.1 Cell-versus-outside counts

lysosome_counts_cell_vs_outside.csv

A grouped summary of the complete lysosome table by location_ch2.

| Column | Type / unit | Meaning |
| --- | --- | --- |
| location_ch2 | text | Classification category, normally “cell” or “outside”. |
| count | integer | Number of lysosome rows in that classification category. |

## 3.2 Counts by original cell

lysosome_counts_by_cell.csv

Counts only lysosomes classified as intracellular, grouped by the original watershed cell ID.

| Column | Type / unit | Meaning |
| --- | --- | --- |
| cell_id_ch2 | integer | Original watershed cell identifier. |
| count | integer | Number of intracellular lysosomes assigned to that cell. |

## 3.3 Counts and volume by serial cell

lysosome_counts_by_cell_serial.csv

Per-cell summary combining lysosome count, original cell ID, serial visualization ID, and physical cell volume.

| Column | Type / unit | Meaning |
| --- | --- | --- |
| cell_id_ch2 | integer | Original watershed cell identifier. |
| lysosomes_in_cell | integer | Maximum consecutive lys_id_in_cell for the cell; because IDs are assigned consecutively, this equals the number of assigned lysosomes. |
| cell_id_serial | integer | Serial visualization cell ID mapped from cell_id_ch2. |
| cell_volume_um3 | µm³ | Physical volume of the serial visualization cell, computed from its voxel count × voxel volume. |

# 4. Cell-level signal datasets

## 4.1 Extended cell signal vs lysosome signal

cell_signal_vs_lysosome_signal_extended.csv

One row per cell. The table partitions cell signal and volume into lysosome core, core-plus-associated, residual membrane, and five distance-to-puncta intervals.

| Column | Type / unit | Meaning |
| --- | --- | --- |
| cell_id | integer | Original watershed cell ID used for the cell-level quantification row. |
| cell_signal_total | CH1 signal sum | Total corrected channel-1 signal within the quantified cell mask. |
| cell_volume_um3 | µm³ | Physical volume of the quantified cell region. |
| lysosome_core_signal | CH1 signal sum | CH1 signal assigned to lysosome core regions within the cell. |
| lysosome_core_volume_um3 | µm³ | Physical volume occupied by lysosome core regions in the cell. |
| puncta_core_fraction | fraction | Fraction of the cell’s total signal assigned to lysosome core signal, as calculated by the script. |
| lysosome_associated_signal | CH1 signal sum | CH1 signal in the core-plus-associated lysosome region. |
| lysosome_associated_volume_um3 | µm³ | Physical volume of the core-plus-associated lysosome region. |
| puncta_associated_fraction | fraction | Fraction of total cell signal assigned to the core-plus-associated lysosome region. |
| membrane_residual_magenta_signal | CH1 signal sum | Residual membrane-associated CH1/magenta signal after the lysosome-associated contribution used by the script is accounted for. |
| membrane_residual_volume_um3 | µm³ | Physical volume of the residual membrane region. |
| membrane_residual_mean_magenta | signal per µm³ | Residual membrane signal divided by residual membrane physical volume. The script explicitly notes that this is not the same as mean intensity per voxel. |
| membrane_residual_fraction | fraction | Fraction of total cell signal assigned to residual membrane signal. |
| HA_signal_0_0p2_um_from_puncta | CH1 signal sum | Total channel-1/HA signal in cell voxels 0.0 ≤ distance < 0.2 µm from a lysosome core. |
| volume_0_0p2_um_from_puncta | µm³ | Physical volume of cell voxels 0.0 ≤ distance < 0.2 µm from a lysosome core. |
| mean_HA_0_0p2_um_from_puncta | mean signal/voxel | Mean channel-1/HA signal per voxel in the 0.0 ≤ distance < 0.2 µm distance bin; NaN when the bin contains no voxels. |
| HA_signal_0p2_0p5_um_from_puncta | CH1 signal sum | Total channel-1/HA signal in cell voxels 0.2 ≤ distance < 0.5 µm from a lysosome core. |
| volume_0p2_0p5_um_from_puncta | µm³ | Physical volume of cell voxels 0.2 ≤ distance < 0.5 µm from a lysosome core. |
| mean_HA_0p2_0p5_um_from_puncta | mean signal/voxel | Mean channel-1/HA signal per voxel in the 0.2 ≤ distance < 0.5 µm distance bin; NaN when the bin contains no voxels. |
| HA_signal_0p5_1_um_from_puncta | CH1 signal sum | Total channel-1/HA signal in cell voxels 0.5 ≤ distance < 1.0 µm from a lysosome core. |
| volume_0p5_1_um_from_puncta | µm³ | Physical volume of cell voxels 0.5 ≤ distance < 1.0 µm from a lysosome core. |
| mean_HA_0p5_1_um_from_puncta | mean signal/voxel | Mean channel-1/HA signal per voxel in the 0.5 ≤ distance < 1.0 µm distance bin; NaN when the bin contains no voxels. |
| HA_signal_1_2_um_from_puncta | CH1 signal sum | Total channel-1/HA signal in cell voxels 1.0 ≤ distance < 2.0 µm from a lysosome core. |
| volume_1_2_um_from_puncta | µm³ | Physical volume of cell voxels 1.0 ≤ distance < 2.0 µm from a lysosome core. |
| mean_HA_1_2_um_from_puncta | mean signal/voxel | Mean channel-1/HA signal per voxel in the 1.0 ≤ distance < 2.0 µm distance bin; NaN when the bin contains no voxels. |
| HA_signal_2_plus_um_from_puncta | CH1 signal sum | Total channel-1/HA signal in cell voxels distance ≥ 2.0 µm from a lysosome core. |
| volume_2_plus_um_from_puncta | µm³ | Physical volume of cell voxels distance ≥ 2.0 µm from a lysosome core. |
| mean_HA_2_plus_um_from_puncta | mean signal/voxel | Mean channel-1/HA signal per voxel in the distance ≥ 2.0 µm distance bin; NaN when the bin contains no voxels. |

Important: Distance bins are defined in the script as [0,0.2), [0.2,0.5), [0.5,1.0), [1.0,2.0), and ≥2.0 µm. Decimal points are replaced by “p” in generated column names.

## 4.2 CH1 signal percentage inside mask

cell_CH1_signal_percent_inside_mask.csv

One row per watershed cell. It compares corrected CH1 signal in the quantified cell region with the portion that also lies inside the original neuron mask.

| Column | Type / unit | Meaning |
| --- | --- | --- |
| cell_id | integer | Original watershed cell identifier. |
| cell_signal_total_CH1 | CH1 signal sum | Total corrected CH1 signal in the quantified cell region. |
| signal_inside_mask_CH1 | CH1 signal sum | Corrected CH1 signal in voxels belonging to both the quantified cell and the original neuron mask. |
| signal_percent_inside_mask | percent | 100 × signal_inside_mask_CH1 / cell_signal_total_CH1. The script returns 0 when total cell signal is zero. |

# 5. How the datasets relate

Detection → lysosome_blobs_regions.csv: establishes lysosome position, radius/diameter, spherical volume, CH1 peak intensity, and peak slice.

Radius uniqueness → lysosome_blobs_regions_unique_radius.csv: optional numerical de-duplication of radii, followed by recalculation of diameter and volume.

Cell segmentation/classification → lysosomes_with_cell_vs_outside.csv: adds cell/outside status and cell/within-cell IDs.

Summaries → lysosome_counts_cell_vs_outside.csv, lysosome_counts_by_cell.csv, and lysosome_counts_by_cell_serial.csv: aggregate the lysosome-level classifications.

Signal quantification → cell_signal_vs_lysosome_signal_extended.csv and cell_CH1_signal_percent_inside_mask.csv: move from one-row-per-lysosome to one-row-per-cell measurements.

Diameter filtering → lysosomes_with_cell_vs_outside_diameter_interval.csv: restricts the lysosome table to the user-selected diameter range and renumbers within-cell lysosome IDs for that subset.

Napari review/editing → lysosomes_with_cell_vs_outside_EDITED.csv: preserves the editable lysosome dataset after interactive classification/serial-ID changes.

# 6. Identifier conventions and interpretation cautions

cell_id_ch2 is the original watershed label. It should not be assumed to be a compact serial numbering scheme.

cell_id_ch2_viz and cell_id_serial are visualization/editing identifiers and may differ from the original watershed label.

lys_id_in_cell is assigned consecutively within a cell after sorting by cell ID and physical Z/Y/X coordinates. It is an identifier, not a measured lysosome property.

Rows classified as outside are assigned cell ID 0 in the classification logic; within-cell lysosome serial IDs are also 0 when outside/unassigned.

peak_gray is a channel-1 intensity measurement, whereas radius_um, diameter_um, and volume_um3 are physical-size measurements.

The extended signal table uses both physical-volume-normalized signal (membrane_residual_mean_magenta) and mean signal per voxel in distance bins; these quantities should not be treated as interchangeable.

The script prints some expected output paths even when an empty lysosome DataFrame means a particular classification file may not actually be created.

# 7. Compact reference

| Question / granularity | Dataset |
| --- | --- |
| One row per detected lysosome | lysosome_blobs_regions.csv |
| One row per classified lysosome | lysosomes_with_cell_vs_outside.csv |
| One row per diameter-filtered lysosome | lysosomes_with_cell_vs_outside_diameter_interval.csv |
| One row per edited lysosome | lysosomes_with_cell_vs_outside_EDITED.csv |
| Counts by location | lysosome_counts_cell_vs_outside.csv |
| Counts by original cell | lysosome_counts_by_cell.csv |
| Counts + volume by serial cell | lysosome_counts_by_cell_serial.csv |
| Cell signal partitioning | cell_signal_vs_lysosome_signal_extended.csv |
| CH1 mask coverage per cell | cell_CH1_signal_percent_inside_mask.csv |