# Napari Interface and Layer Guide

Microscopy cell and lysosome workflow - current interface

Figure 1. Current napari workspace, including the two additional derived mask layers: Mask 3 - Mask 1 minus Mask 2 and Mask 1 - membrane from Ch2.

# 1. Quick orientation to the current napari window

Upper-left: layer display controls for the selected Points layer.

Lower-left: the complete layer stack, including the two additional mask layers.

Center: fluorescence, segmentation, masks, and cell:lysosome annotations are composited on the canvas.

Upper-right: Filter cells / lysosomes.

Middle-right: Select lysosomes in the editable layer.

Lower-right: feature table for Lysosomes (EDIT TABLE), with editable toggle and Save as CSV.

# 2. Layers in the current project

| Layer | Type | Role | How to use it |
| --- | --- | --- | --- |
| Lysosomes (VIEW FILTER) | Points | Filtered display subset of lysosome points. | Use after Apply filter for focused viewing. |
| Lysosomes (EDIT TABLE) | Points | Working/editable lysosome point set. | Use for selection, review, and intentional metadata editing. |
| ID (filtered view) | Points/text | ID labels for the filtered subset. | Show with VIEW FILTER. |
| ID (serial) | Points/text | Full serial ID annotation set. | Use for full-field identity context. |
| Cell mask | Labels | Segmented cell region(s). | Use with reduced opacity to check cell boundaries. |
| Ch 03 - membrane minus lysosomes | Derived image | Processed membrane/context signal with lysosome-related regions removed or excluded by the workflow. | Compare with original channels and mask layers for spatial QC. |
| Lysosome CORE outline | Outline | Boundary of core lysosome regions. | Preferred for boundary QC. |
| Lysosome CORE fill | Labels | Filled core lysosome segmentation. | Use with partial opacity. |
| Lysosome ASSOCIATED outline | Outline | Boundary of associated/peripheral lysosome regions. | Compare directly with CORE outline. |
| Lysosome ASSOCIATED fill | Labels | Filled associated/peripheral lysosome regions. | Compare with CORE fill and underlying signal. |
| Mask 3 - Mask 1 minus Mask 2 | Derived mask | Difference mask produced by subtracting Mask 2 from Mask 1. | Use to inspect the portion retained in Mask 1 after removal of the Mask-2 region; compare with Ch 03 and the original channels. |
| Mask 1 - membrane from Ch2 | Derived mask | Membrane mask derived from Channel 2. | Use as the direct segmentation/QC representation of the Channel-2 membrane; compare against Ch2 original - GREEN and Ch2 grayscale. |
| Ch1 original - MAGENTA | Image | Original Channel 1 displayed in magenta. | Primary Channel-1 color reference. |
| Ch2 original - GREEN | Image | Original Channel 2 displayed in green. | Primary Channel-2 color reference. |
| Ch1 grayscale | Image | Grayscale display of Channel 1. | Useful for intensity/morphology without color mixing. |
| Ch2 grayscale | Image | Grayscale display of Channel 2. | Useful for Channel-2 membrane/intensity inspection. |

# 3. The two additional mask layers

## Mask 1 - membrane from Ch2

This layer represents the membrane mask derived from Channel 2. Its main purpose in napari is quality control: turn on Ch2 original - GREEN or Ch2 grayscale underneath it and check whether the mask follows the visible Channel-2 membrane signal. Use partial opacity when needed so both the segmentation and raw fluorescence remain visible.

QC questions: Does the mask follow the true cell membrane? Does it extend into background? Are real membrane regions missing? Does it preserve thin extensions of the cell?

## Mask 3 - Mask 1 minus Mask 2

This is a derived difference mask. The layer name indicates that it contains the region remaining after Mask 2 is removed from Mask 1. In practical QC, it lets you see which part of the Mask-1 membrane region remains after the second mask has been excluded.

Use it together with Mask 1, the lysosome CORE/ASSOCIATED layers, Ch 03 - membrane minus lysosomes, and the original fluorescence. Toggling these layers on and off is the safest way to understand exactly which spatial regions are being retained or removed.

# 4. Recommended display order

Bottom: Ch1 original - MAGENTA and Ch2 original - GREEN, or their grayscale versions.

Above the raw images: Cell mask and Mask 1 - membrane from Ch2 with partial opacity.

Above Mask 1: Mask 3 - Mask 1 minus Mask 2 and other derived contextual layers such as Ch 03.

Above the masks: Lysosome CORE and ASSOCIATED fills/outlines.

Top: Lysosome point layers and ID text layers.

# 5. Filter cells / lysosomes

| Control | Use |
| --- | --- |
| Cell IDs | Enter all, individual IDs, comma-separated IDs, or ranges. |
| Lys IDs in those cells | Optionally restrict lysosomes within selected cells. |
| location | Choose the location/classification scope. |
| Show filtered ID labels layer | Toggle ID (filtered view). |
| Show VIEW FILTER points layer | Toggle Lysosomes (VIEW FILTER). |
| Apply filter | Apply the requested visual subset. |

# 6. Select lysosomes and EDIT TABLE

The selection widget identifies matching records in Lysosomes (EDIT TABLE). Filtering controls what is displayed; selection controls which editable records are selected.

| Feature column | Meaning |
| --- | --- |
| location_ch2 | Channel-2 location/classification. |
| cell_id_serial | Serial cell identifier. |
| lys_id_serial | Serial lysosome identifier. |
| diameter_um | Lysosome diameter in micrometers. |
| peak_gray | Peak grayscale/intensity-related measurement. |
| slice | Associated image-slice index. |

# 7. Updated QC workflow including the mask layers

Start with Ch1 original - MAGENTA and Ch2 original - GREEN.

Check Cell mask against the original fluorescence.

Turn on Mask 1 - membrane from Ch2 and compare it directly with Channel 2.

Toggle Mask 3 - Mask 1 minus Mask 2 to inspect the region remaining after the Mask-2 exclusion.

Compare Mask 3 with Ch 03 - membrane minus lysosomes to understand the derived membrane/residual region.

Inspect Lysosome CORE outline and fill.

Inspect Lysosome ASSOCIATED outline and fill.

Use ID (serial) to confirm cell:lysosome identities.

Filter to a manageable subset when needed.

Cross-check cell_id_serial and lys_id_serial in the EDIT TABLE.

Select/edit only when necessary and re-check the masks and original channels afterward.

Save as CSV after validation and Clear filter before a new query.

# 8. Quick reference

| Goal | Layer(s) to use | What to check |
| --- | --- | --- |
| Original Channel 1 | Ch1 original - MAGENTA / Ch1 grayscale | Raw lysosome-related intensity and morphology. |
| Original Channel 2 | Ch2 original - GREEN / Ch2 grayscale | Raw membrane/cell-related intensity. |
| Cell segmentation | Cell mask | Cell boundary against fluorescence. |
| Channel-2 membrane segmentation | Mask 1 - membrane from Ch2 | Agreement between membrane mask and raw Ch2. |
| Difference region | Mask 3 - Mask 1 minus Mask 2 | What remains from Mask 1 after Mask 2 is removed. |
| Derived membrane residual | Ch 03 - membrane minus lysosomes | Relationship between residual membrane signal and lysosome regions. |
| Lysosome core | CORE outline / fill | Core boundary and measured region. |
| Associated lysosome region | ASSOCIATED outline / fill | Expanded/peripheral region relative to CORE. |
| IDs | ID serial / filtered view | Correct cell:lysosome identity. |
| Measurements | Lysosomes (EDIT TABLE) | IDs, location, diameter, peak intensity and slice. |